# SPOT Program Development Report (Structure)

Introduction

The current construction monitoring and inspection is manual process and has led to inefficient contributions to cost overruns in 66% of construction project and schedules delays by 53% in projects, affecting almost $1.4 trillion worths of structures being constructed each year in United States [1].

The absence of real time information handicaps manager’s ability to monitor schedule, cost and other performance indicators, which reduces their ability to detect or manage the variability and uncertainty inherent in project activities. A significant portion of the inspection time (30-50%) is spent on visual data collection of the as-built status and on-site analysis[2].

The majority of the time in meetings is spent on descriptive (35%) and explanative (42%) tasks and only 12% and 11% of the time is spent on evaluative and predictive tasks respectively. One of the major reasons for this, is lack of adequate means to visualize and represent the information[3]. Also, the quality of inspection data is also determined by the inspector's experience and the methods used to collect measurements [1].

Previous studies investigated automating construction inspection. These studies aimed to increase the frequency and accuracy of inspection by capturing visual data using semi-autonomous or autonomous systems. These studies either focused primarily on the data capture alone and not on presenting the information in a way that is intuitive for inspection or used heavy post-processing that requires skilled personnel with specialized training. Another challenge with heavy post-processing of inspection data (i.e. as-built reality capture) is the delay in the availability of inspection data. By the time the reality capture is available to inspectors and project stakeholders, the construction work might have progressed further, and either the identified construction issues might be resolved already, or new issues might have arisen [4].

The key in progress monitoring is accurate, timely, and regular data collection and analysis during the construction process. In fact, automated progress monitoring of construction projects using emerging technologies can address existing limitations and inefficiencies of the manual processes and can enable systematic recording and analyzing construction progress. Identified four areas in automating the process of progress monitoring: (a) data collection from the actual construction work, (b) extracting required as-built data for analysis, (c)comparing the as-built data with as planned information and (d) visualizing and reporting the results. 
Specifically in order to automate data collection in this process, studies have used various technologies, for example, a combination of Radio-Frequency Identification (RFID), laser scanning, photogrammetry with a digital camera, barcoding and a mobile device, a combination of camera-equipped UAV (Unmanned Aerial Vehicles), 4D Building Information Modeling (BIM), and 3D point cloud model and a data fusion framework that combines multiple data sources including 3D CAD model, ultra-wideband (UWB) receivers data, and laser scanned data. In this regard, collecting photo images of the actual construction status is very important for the reporting stage and for supporting communication of the identified issues with the stakeholders. 360° cameras are being utilized in these applications to increase visibility and decrease the capture time. Recent advances in automation have enabled robotic and autonomous inspection and monitoring. Mobile robots can be deployed to release the human from the laborious task of constant on-site image capture.
Autonomous navigation through the unstructured and dynamically changing construction environments requires mobile robots that can carry out high-performance locomotion tasks. Mobile robots use various types of locomotion to navigate an environment. Typical locomotion types used for mobile robots are : (a) wheeled robots using multiple wheels for navigation as the most stable robots with high payload capacity but they can operate on limited uneven surfaces and cannot move between levels through the stairs independently, (b) crawler-mounted robots using crawlers with continuous tread bases that can operate on unlevelled and unfinished ground, (c) climbing robots using suction mounts or magnetic adhesion to stick to walls that are typically used for façade inspection but their challenge is to stay affixed to the surface reliably while being able to avoid obstacles, (d) Unmanned Marine Vehicles (UMV) mechanically sealed to operate in water and typically used for underwater inspection with special sensing devices, (e) Unmanned Aerial Vehicles (UAV) using rotors to fly and more suited for outdoors but in an enclosed construction environment there is a potential safety risk for workers and they may even collide with building materials, columns, or overhead cables, (f) legged robots using one or multiple legs inspired from terrestrial mammals to move but maintaining balance while moving is a major challenge for legged robots, (g) hybrid robots using a combination of locomotion systems e.g. a leg-wheel robot. 
Legged robots increase the field of different accessible terrains and are more versatile than wheeled robots providing good potential to traverse construction sites. Mostly used and developed legged robots are two-legged robots, biped or humanoid robots, four-legged robots or quadruped, and six-legged robots or hexapod. Legged robots have become an important research direction in the field of robotics in recent years. Numerous legged robots have been developed by companies, institutions, and individuals including NASA, MIT, IIT, ETH, Boston Dynamics, Ghost Robotics, ANYbotics, etc. But due to high technological complexity to build and control such robots there are only few teams who have developed legged robots that can be applied outside laboratory settings. Multi-legged robots can achieve better locomotion performance in terms of speed, efficiency of movement, and obstacle avoidance. Legged robots have better adaptability on rough terrains, better mobility and flexibility because of many Degrees of Freedom (DoF) of the legs and can move their body independent of underlying terrain making them a good potential for traversing construction environments[5].

Quadruped robots have a wider range of accessible terrains and are more versatile than wheeled robots, including better mobility, stability, and flexibility due to having more Degrees of Freedom (DoF) per leg. Quadruped robots can traverse uneven terrains, move up and down stairs, and walk over small obstacles, making them more suitable for the construction environment. Quadruped robots can assist humans on job sites in construction inspection and monitoring, but they also require periodic human intervention on unstructured and dynamically changing construction sites [1].

## 1.1 Context and Motivation

#### Why SPOT as a quadruped robot?

- Handles stairs and irregular terrain by design

- Can climb ±30° slopes

- 300 mm step height

- IP54 protection for dust/splash, temperature range -20°C to 45°C

- 360° Obstacle avoidance

- Reliable travels through doors and corridors, where wheels could get stuck or skid.

## 1.2 Objectives

- To develop a mixed control framework for construction site inspection with human operator and SPOT collaborate to improve efficiency, consistency and safety of monitoring process.

- To design an integrated workflow collaborated inspection using a fiducial marker detection system.

- To evaluate the effect of collaborated control system on operator and navigational efficiency.

- To measure how this system performs compared to fully manual or fully autonomous approach.

- Does it reduce operator’s workload?

- Does it allow for faster navigation through complex areas?

- To help build an understanding of the human-robot team dynamics.

- To compare collaborated control inspection verses manual inspection.

- At a tagged location, perform pose-hold + 360° Photo + audio note.

# 2. System Overview

## 2.1 Hardware

- About SPOT

| Specification | Value |
| --- | --- |
| Length | 1100 mm |
| Width | 500 mm |
| Height (standing) | 840 mm |
| Height (Seating) | 191 mm |
| Weight | 32.5 kg |
| DOF | 12 |
| Speed | 1.6 m/s |
| Ingress protection | IP54 |
| Operating Temperature | -20 C to 45 C |
| Slopes | +/- 30 Degrees |
| Max step height | 300 mm |
| Lighting | Above 2 lux |
| Battery capacity | 605 Wh |
| Max battery voltage | 58.8V |
| Typical runtime | 90 minutes |
| Standby runtime | 180 minutes |
| Charger power | 400W |
| Max charge current | 7A |
| Time to charge | 120 minutes |
| Payload max weight | 14 kg |
| Max power per port | 150 W |
| Payload ports | 2 |
| Camera type | Projected stereo |
| Field of view | 360 Degrees |
| Operating range | 4 meters |
| Wifi | 802.11 |
| Ethernet | 1000Base-T |

Cameras

- Spot exposes five fisheye cameras and five depth cameras as image sources via the on-robot Image service, enabling multi-camera capture with hardware time-sync when multiple sources are requested in a single RPC.

- Payload Integration (Ricoh Theta)

## 2.2 Software and API’s

#### OS, Python, SDK version

- The SDK Python client supports Ubuntu, Windows, and macOS with Python 3.7–3.10 and includes QuickStart, examples, and full API docs for rapid development.

- Libraries

#### Services

- Core services to use: Directory and directory-registration, auth, robot-state, robot-command, image, lease, estop, and autonomy services (e.g., GraphNav) are discoverable via the directory service and are listed by default on a running robot.

- Time sync: The Time Sync service coordinates clock skew and ensures command validity windows; the client maintains a background timesync thread after wait_for_sync

# 3. Methods and Implementation

## 3.1 Machine Control

- Control prerequisites: Before mobility commands, authenticate, configure E‑Stop if used, acquire a lease, establish time sync, then power on; use RobotCommand service to stand, sit, velocity, and trajectory commands.

- Leases: A valid Lease is required on every mobility command and must be actively kept alive; the “body” lease covers mobility and related sub-resources, and a keepalive must persist for the duration of control.

- Power control: Use power_on to bring actuators online and power_off with cut_immediately=False to sit before shutdown, as recommended for safe operation.

- Manual “WASD” mapping: Keyboard teleop can be implemented by sending velocity and trajectory commands via RobotCommandClient while holding a valid lease and tracking command feedback.

- Different modes: Manual mode (WASD), Autonomous mode (Following Tag)

- Keyboard Input : toggle menu

## 3.2 Perception : AprilTag/Fiducial Detection

Image acquisition: Use ImageClient to list camera sources and request RAW or JPEG images; multiple sources in one request are hardware time-synced for better multi-camera detection consistency.

External fiducials: AprilTag detection is implemented application-side using images from the Image service; for slow-to-capture sensors, implement a Data Acquisition plugin instead of an Image service to avoid blocking expectations.

Orientation handling: Normalize camera orientation in preprocessing using the known rotations of specific cameras to improve tag detection pipelines.

- Cameras, Image procesing

- AprilTag family

- Tag ID detection, Camera output

## 3.3 Safety

- Toggle power

- Motor on/off

- Toggle lease

#### E-Stop

E‑Stop service requires endpoints to register and check in; stop levels CUT, SETTLE_THEN_CUT, and NONE determine actuator behavior,

## 3.4 Payload integration (Optional)

- Ricoh Theta

- Payload registration and authentication

- Live preview stream and capture

## 3.5 Logging and diagnostics

- Structured logging: The SDK examples rely on Python logging and include verbose client tracing that shows service calls, headers, and timing; adopting the same logging makes debugging consistent with SDK tooling.

- Single line status update

- Updates: Tag detected/ID, RPC time out errors

# 4. Experiments

## 4.1 Test scenarios

## 4.2 Metrics and Measurements

- Detection latency, frame rate

## 4.3 Procedure

- Step by  step testing, data collection

## 5. Results & Discussion

- Camera stream errors, RPC timeout

- Camera input, wrong follow direction

- Wrong posing while following

- Simultaneous Manual and autonomous operation

- Live payload streaming (Ricoh theta)

## 6. Conclusion

- What worked, what didn’t, and the practical impact.

- Multitag selection

- Stairs follow mode integration

- Intergration with external device (x-box controller)

### 7.How to Operate SPOT
7.1 Initiation Checklist

- Clear the area & brief people nearby.

- Ensure an operator has a reachable hardware or software E-Stop.

- Power and battery check.

- Put the tablet or your control laptop on the same network as the robot.

### 7.2 Programming Operations

- Install the Python SDK

  - Install Python 3.7–3.10.

  - Verify your python install is the correct version. Open a command prompt or start your python IDE:

Then enter

$ python --version
	Python 3.10.12

  - Windows users: There are two common methods for starting the

Python interpreter on the command line. First, launch a terminal: Start > Command Prompt. At the command prompt, enter either:

> python.exe

Or

> py.exe

- Pip Installation

Pip is the package installer for Python. The Spot SDK and the third-party packages used by many of its programming examples use pip to install.

Check if pip is installed by requesting its version:

$ python3 -m pip --version
	pip 19.2.1 from <PATH_ON_YOUR_COMPUTER>

For Windows users:
	> py.exe -3 -m pip –version

- Install Spot Python packages

With python and pip properly installed and configured, the Python packages are easily installed or upgraded from PyPI with the following command:

$ python3 -m pip install --upgrade bosdyn-client bosdyn-mission bosdyn-choreography-client bosdyn-orbit

Installing the bosdyn-client, bosdyn-choreography-client and bosdyn-mission packages will also install bosdyn-api and bosdyn-core packages with the same version. The command above installs the latest version of the packages.

- Verify your Spot packages installation

Enter this command to verify all the packages

$ python3 -m pip list --format=columns | grep bosdyn

The Output Should be :
bosdyn-api                    5.0.1
bosdyn-choreography-client    5.0.1
bosdyn-choreography-protos    5.0.1
bosdyn-client                 5.0.1
bosdyn-core                   5.0.1
bosdyn-mission                5.0.1
bosdyn-orbit                  5.0.1

For Windows Users:

> python3 -m pip list --format=columns | findstr bosdyn

For more accurate step and troubleshooting you can refer to: https://dev.bostondynamics.com/docs/python/quickstart

Sources

[1] Halder, Srijeet, et al. "Construction inspection & monitoring with quadruped robots in future human-robot teaming: A preliminary study." Journal of Building Engineering 65 (2023): 105814.

[2] Navon, Ronie, and Rafael Sacks. "Assessing research issues in automated project performance control (APPC)." Automation in construction 16.4 (2007): 474-484.

[3] Kopsida, Marianna, Ioannis Brilakis, and Patricio Antonio Vela. "A review of automated construction progress monitoring and inspection methods." Proc. of the 32nd CIB W78 Conference 2015. 2015.

[4] Halder, Srijeet, et al. "A methodology for BIM-enabled automated reality capture in construction inspection with quadruped robots." ISARC. Proceedings of the International Symposium on Automation and Robotics in Construction. Vol. 38. IAARC Publications, 2021.

[5]  Afsari, Kereshmeh, et al. "Fundamentals and prospects of four-legged robot application in construction progress monitoring." EPiC series in built environment 2 (2021): 274-283.